###### RELIABILITY OF CONSECUTIVELY CONNECTED SYSTEMS

## *

## GEORGE K. PAPADOPOULOS

Counsellor, Hellenic Pedagogical Institute

Abstract

In the present paper we use the Markov chain approach to study two reliability systems that have recently attracted special interest, namely: Consecutively-Connected Systems and Multistate Consecutively-Connected Systems. More precisely, a proper state space and finite Markov chain is introduced to describe each of these systems and subsequently exact formulas are derived for their reliability.

Περίληψη

Στην παρούσα εργασία μελετάμε την αξιοπιστία δυο συστημάτων για τα οποία υπάρχει έντονο ερευνητικό ενδιαφέρον και ευρύ φάσμα εφαρμογών: Συνεχόμενα - Συνδεδεμένα Συστήματα (CCS) και Συνεχόμενα - Συνδεδεμένα Συστήματα πολλών καταστάσεων (MCCS). Συγκεκριμένα, περιγράφουμε καθένα από τα συστήματα αυτά με κατάλληλες Μαρκοβιανές αλυσίδες , τις οποίες ορίζουμε, και δίνουμε ακριβείς τύπους για τον υπολογισμό της αξιοπιστίας τους

Key Words

Consecutively-Connected System, Multistate Consecutively-Connected System, Consecutive-k-out-of-n:F systems, Markov chains, Markov Imbedding, Transition probability, Reliability

### Reader Aids

General purpose: Present a new result

Special math needed for explanations: Probability theory

Results useful to: Reliability analysts and designers

1.	Introduction

The Consecutively-Connected Systems can be applied in the design of electronic devices, model spacecraft relay stations, telecommunication and oil pipeline systems, computer networks and vacuum systems in accelerators.

The first system in this category was introduced by Kontoleon [2] and it is commonly known as consecutive-k-out-of-n: F system. It consists of an ordered sequence of n s-independent components; all of them have same transmitting capability k. Also, each component and the system are either working or failed. The system fails iff at least k consecutive components fail. For example, a telecommunications system with n relay stations, where each working station can transmit a signal to the next k stations, may be modelled as a consecutive k-out-of-n: F system. The consecutive-k-out-of-n: F system has two limitations which limit to a significant extent the range of its application

1.	All components have the same transmitting capability k

2.	Each component has only two states, i.e. working-failed.

In order to overcome not only the above but also other limitations, a lot of modifications, extensions and generalizations have been made as far as the principle of operation (or breakdown policy) and the arrangement of its componets are concerned (a review concerning the above can found at Papastavridis & Koutras [4])

Shanthikumar [5] extended the consecutive k-out-of-n: F system to the Consecutively-Connected Systems (CCS) whose components do not have the same transmitting capability, whereas Hwang and Yao [1] extended the CCS to the Multistate Consecutively-Connected Systems (MCCS) whose components are not necessarily 2-state.

In order to calculate the reliability of CCS and MCCS several algorithms have been introduced by Shanthikumar [5] and Zuo [6] and Hwang & Yao [1] correspondingly. Koutras [3] has introduced a unified framework for reliability structures which can be described by finite Markov chains called Markov Chain Imbeddable Reliability Structures. In this paper we prove that both MCCS and CCS can be described by finite Markov chains and by applying the general result of Koutras [3] we present exact formulas for their reliability.

#### Notation

| n | number of components |
| --- | --- |
| kj | transmitting capability of component j or maximum number of components directly connected to component j |
| k | max kj0≤j≤n |
| m | min j+kj, n+1 |
| pt, qt=1−pt | [reliability, unreliability] of component t |
| S | s0,s1, …, sN, a finite state space |
| Yt, t=0, 1, 2,… | a Markov chain defined on S |
| π0 | (P [ Y0=s0], P [ Y0=s1], ..., P [ Y0=sN])' the vector of initial probabilities |
| pij(t) | P [Yt=sj ¦ Yt−1=si], t≥1, transition probabilities |
| Λt | transition probabilities matrix |
| u | (1, 1, ..., 1, 0)1x(N+1)' |
| Rn | reliability of a system |
| CCS | Consecutively-Connected System |
| MCCS | Multistate Consecutively-Connected System |

2. Reliability of CCS

The CCS consists of a source (0), n s-independent components  1, 2, …, n and a sink n+1. The source is connected to components  1, 2, …, k0 and components j 1≤j≤n are connected to components  j+1, j+2, …, m by arcs. That is to say, if the component j, 1≤j≤n functions it has direct access to the next kj components. The source, the sink, and the arcs are perfect whereas the components have two states, functions or failed. The system functions iff there is a connection from the source to the sink through functioning components. Obviously CCS allows the modelling of systems in which different components have different transmitting capabilities. If kj=k for all j=0, 1, …, n, then the CCS is the consecutive k-out-of-n: F system.

We consider the state space S= 0, 1, …, k which can be partitioned as S=i=0k i. Let Yt=i, i=0, 1, ..., k if and only if the component t+k−i of the system is accessible through working components of the subsystem 0, 1, ..., t. Then, the transition probabilities pij(t) are

1.	Pk,jt=δk,j

2.	Pk−1,jt=pt if j=k−kt

3.	For 0≤i≤k−1 we have the following two cases

- if kt≤k−i+1 Pi,jt=δj,i+1

- if kt>k−i+1 Pi,jt=qt for j=i+1

and the reliability of the system is given by Rn=π0'(t=1nΛt) u (Koutras [3]).

## EXAMPLE 1

1

2

3

4

5

6

7

0

k0=2

k2=1

k3=1

k1=4

k6=1

k4=3

k5=2

Let us consider a CCS as sketched in Fig. 1 with transmitting capabilities k0=k5=2, k1=4, k2=k3=k6=1, k4=3

Figure 1

The state space of the system is S= 0, 1, 2, 3, 4 and the 5×5 transition probability matrixes are

Λ1=p1q1000p10q100p100q10p1000q100001			Λi=010000010000010000piqi00001 i=2, 3, 6

Λ4=010000p4q4000p40q400p400q400001		Λ5=010000010000p5q5000p50q500001.

Then the reliability of the system is given by R=π0' Λt u, where π0=(0, 0, 1, 0, 0)' and u=(1, 1, 1, 1, 0)'.

3. Reliability of MCCS

##### Additional notations

| ##### ℓ | ##### min i+j, n+1 |
| --- | --- |
| ##### Zi | ##### a discrete random variable which describe the state of component i |
| ##### pi,j | ##### P Zi=j |

The MCCS consists of a source (0), n s-independent and multistate components  1, 2, …, n and a sink n+1.The source, the sink, and the arcs are perfect whereas the component i, i=0, 1, …, n can be in one of the states 0, 1, …, ki with correspondiny probabilities pi,0, pi,1, …, pi,ki. Then the state of the component i  i=0, 1, …, n will be described by a discrete random variable Zi with probability distribution P Zi=j=pi,j j=0, 1, …, ki,  pi,0+pi,1+ ⋯ +pi,ki=1. If the component i is in the state 0 Zi=0 then it is failed, while if it is in the state j Zi=j then there are arcs from component i  i=0, 1, …, n to components i+1, i+2, …ℓ and there aren’t arcs to componets i+j+1, …, i+j+ki. The system functions iff there is a connection from the source to the sink through functioning components. Obviously MCCS allows the modelling of systems in which different components have more than two states. For example a system of relay stations where each of the stations i  i=0, 1, …, n consists of ki amplifiers. If all of the amplifiers of a station i are functioning then a signal from i reaches the next ki stations (or the sink if i+ki≥n+1). The failure of one amplifier reduces the transmitting capability of the station to which it belongs by one i.e. a signal from that station reaches the next ki−1, and so on. If pij=0, j=1, …, ki−1 then the MCCS is the CCS with pi=pi,ki and qi=pi,0.

We consider the state space S= 0, 1, …,k, which can be partitioned as S=i=0k i. Let Yt=i, i=0, 1, …, k if and only if the component t+k−i of the system is accessible through working components of the subsystem 0, 1, …, t. Then, the transition probabilities pijt are

1.	Pk,jt=δk,j

2.	Pk−1,jt=pt,k−j for k−kt≤j≤k

3.	For 0≤i≤k−1 we have

- if kt≤k−i+1 then Pi,jt=δi+1,j

- if kt>k−i+1 then Pi,jt= pt,s for j=i+1

and the reliability of the system is given by Rn=π0' t=1nΛt u where π0=(0, 0, 1, 0, 0)' and u=(1, 1, 1, 1, 0)'.

## EXAMPLE 2

Let us consider a MCCS as sketched in Fig. 1. The state space of the system is S= 0, 1, 2, 3, 4 and the 5×5 transition probability matrixes are

Λ1=p14p13+p12+p11+p10000p14p13p12+p11+p1000p14p13p12p11+p100p14p13p12p11p1000001

Λi=010000010000010000pi0pi000001 i=2,3,6

Λ4=010000p43p42+p41+p40000p43p42p41+p4000p43p42p41p4000001

Λ5=010000010000p52p51+p50000p52p51p5000001.

Then the reliability of the system is given by R=π0' Λt u, where π0=(0, 0, 1, 0, 0)' and u=(1, 1, 1, 1, 0)'.

References

HWANG, F. K., YAO, Y. C. «Multistate consecutively connected systems», IEEE Trans. Reliability, (1989), 38 (4), pp. 472-474.

KONTOLEON, J. M. «Reliability determination of a r-successive-out-of-n:F system», IEEE Trans. Reliability, (1980), 29, pp. 437.

KOUTRAS, M. V. «On a Markov chain approach for the study of reliability structures», JAP, (1996), 33, pp (to appear).

PAPASTAVRIDIS, S. G., KOUTRAS, M. V. «Consecutive-k-out-of-n systems», New trends in system reliability evaluation, (MISRA, K. B. Ed), Elsevier / North Holland, Amsterdam, (1993), pp. 228-248.

SHANTHIKUMAR, J. G. «Reliability of systems with consecutive minimal cutsets», IEEE Trans. Reliability, (1987), 36 (5), pp. 546-550.

ZUO, M. «Reliability of linear and Circular consecutively connected systems», IEEE Trans. Reliability, (1993), 4(3), pp. 484-487.

*Manuscript received, February 23, 2000